HANDY-ROM  User's Manual

1. HANDY-ROM

HANDY-ROM is a small sized portable PMC-WRITER .

HANDY-ROM is designed for IBM PC users and all instructions are same with  FANUC  PMC-WRITER..

2. Connecting IBM -PC/AT( 9 Pin) to Handy-ROM

IBM-PC/AT                                                 HANDY

- ROM

COM1(DB9P)                  Cable

3. EPROM

HANDY-ROM support  FANUC  SYSTEM 0 M/T.

Users can use EPROM 256K BIT( 27C256 ). Vpp = 12 Volt

4. HANDY-ROM  Manipulation

HANDY-ROM does not have any setting switch or data input button.

You just can manipulate the HANDY-ROM with the FLADDER software program through RS-232. Before you start,

1)Connect HANDY-ROM to PC with the cable.

2)Connect DC Adapter to HANDY-ROM.

3)Turn on the Switch .

(1)  CONFIG . SYS

System use ANSI’s Escape Sequence, so PC DOS system must set  ANSI.SYS in CONFIG..SYS

DEVICE = A : \ DEV \ ANSI . SYS

- AUTOEXEC.BAT

If your system use Bi language then must chage English mode.

hcode/e

(3)  EMS

MS-DOS  VER. 5.0 / 6.2

DEVICE = C : \ DOS \ HIMEM.SYS

DEVICE = C : \ DOS \ EMM386.EXE FRAME = C000

FLADDER

In DOS Mode, type FLADDER and hit the ENTER Key.

FAPT  LADDER

A 0 8 B - 9 2 0 1 - J 5 0 0 # E N

Ver  0 6 . 3       １９９３.１０.２３

Ｃｏｐｙｒｉｇｈｔ(Ｃ）０９９０　ＦＡＮＵＣ　ＬＴＤ

ＨＩＴ　ａｎｙ　ｋｅｙ　！

Press any key, then the main page will appear.

At the main page, you can select  FUNCTION key.

MAIN MEＮＵ PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 F1 KEY : EDIT F2 KEY : PRINT F3 KEY : COMPILE F4 KEY : DISCOMPILE F5 KEY : UNUSED F6 KEY : I / O F7 KEY : SETUP F8 KEY : NEMONIC EDIT F9 KEY : UTILITY F10 KEY : END 1EDIT 2PRINT 3COMPIL 4DISCPL 5 6 I/O 7SETUP 8MNEEDT 9UTILITY 10END

If you named the ladder program name for EPROM Write and Verify, go the I/O Mode by F6 Key.

If not, go to the Edit mode and give the file name by F1 Key.

EDIT ( SELECT SOURCE PROGRAM )PMC-Ｌ <0> (Ｃ:\ FLADDER\ 〕 1) INPUT PROGRAM NAME : TESTPROG 2) OUTPUT PROGRAM NAME : TESTPROG 1EXEC 2 3 4 5 6 7 8 9 10END

Before you press  F2 key, turn on the Handy- ROM power and connect HANDY-ROM to COM1 port with Handy-ROM connector.

MAIN MEＮＵ PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 F1 KEY : FA WRITER F2 KEY : PMC WRITER F3 KEY : PMC F4 KEY : HANDY FILE F5 KEY : UNUSED F10 KEY : END 1 FAWRT 2 PMCWRT 3 PMC 4 FDCAS 5 6 7 8 9 10END

Hit the F2 key.

SERIAL  PORT  PARAMETER  =  COM1  48  N  8  2

MAIN MEＮＵ PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 F1 KEY : READ F2 KEY : WRITE F3 KEY : VERIFY F4 KEY : BLANK CHECK F10 KEY : END 1 READ 2 WRITE 3 VERIFY 4 BLANK 5 6 7 8 9 10END

EPROM BLANK CHECK

Please use new EPROM or erased EPROM.

I / O ( WRITER ) PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 EXCUTE FORM : BLANK CHECK ROM KIND : 256KBIT ROM SERIAL PORT PARAMETERS = COM1 48 N 8 2 NORMAL END 1 2 3 4 5 6 7 8 9 10END

Nomal Blank checked EPROM

I / O ( WRITER ) PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 EXCUTE FORM : BLANK CHECK ROM KIND : 256KBIT ROM SERIAL PORT PARAMETERS = COM1 48 N 8 2 BLANK CHECK ERROR ADDRESS = 000000 , LOW = 00 , DATA = 61 1 2 3 4 5 6 7 8 9 10END

Not Erased EPROM

START

Turn on HANDY - ROM Power

Set the EPROM(27C256) at TEXTOOL

Hit the “F4” KEY

BLANK TEST

END

EPROM  WRITING

FANUC  SYSTEM 0  Series PMC have two EPROM (27265) , 0E1 and 0E2.

ROM Writing for a EPROM will take about 7 minutes and show ADDRESS ( 0000 ~ 7FFF  16 bit)

If the EPROM is not erased, ERROR MESSAGE will be appear with address.

Normally you can see ADDRESS = 07FFF.

After EPROM  0E1 writing, select 0E2 by right cursor and put the new EPROM at Handy-ROM for Writing.

Red LED will be blink during its writing.

I / O ( WRITER ) PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 EXCUTE FORM : WRITE ( PROGRAMMER PMC WRITER ) ROM KIND : 256KBIT ROM ROM FORM FILE NAME : TESTPROG.#OE ROM NO. : OE1 OE2 ADDRESS = 007fff SERIAL PORT PARAMETERS = COM1 48 N 8 2 1 Write 2 3 4 5 6 7 8 9 10END

( 1) FLADDER for IBM PC does not check if the EPROM is cleared during EPROM Writing.

So, before writing, you must check if the EPROM is clean.

( 2)  HANDY-ROM can not program EPROM 21.5Volt type.

START

Select EPROM WRITE Mode

Check the file name

Select ROM  ( 0E1 )

Set the clear EPROM on

HANDY-ROM

EPROM WRITE

NO

OK ?

Select ROM  ( 0E2 )

Set the clear EPROM on

HANDY-ROM

EPROM WRITE

NO

OK ?

YES

END

EPROM  VERIFY

“EXECUTE” will blink at the PC and Red LED at  the HANDY - ROM will blink during Verify.

I / O ( WRITER ) PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 EXCUTE FORM : VERIFY ( PROGRAMMER PMC WRITER ) ROM KIND : 256KBIT ROM ROM FORM FILE NAME : TESTPROG.#OE ROM NO. : OE1 OE2 ADDRESS = 0004bf EXECUTING SERIAL PORT PARAMETERS = COM1 48 N 8 2 1 2 3 4 5 6 7 8 9 10END

Reading the EPROM data.(READ)

During the EPROM READ, “EXECUTE” will blink and will show the ADDRESS.

Red LED  of  HANDY - ROM will blink.

I / O ( WRITER ) PMC-Ｌ 〔Ｃ:\ FLADDER\ 〕 EXCUTE FORM : READ ( PROGRAMMER PMC WRITER ) ROM KIND : 256KBIT ROM ROM FORM FILE NAME : BUIL-TEST ROM NO. : OE1 OE2 ADDRESS = 007fff SERIAL PORT PARAMETERS = COM1 48 N 8 2 SET ANOTHER ROM, AND SELECT ROM NO. 1 2 3 4 5 6 7 8 9 10END

BUIL Electric co.

C-18 P1-FL. Main-BD. ET-Land

16-9 HanGangno-3ga YongSan-ku

Seoul 140-879 South KOREA

TEL: +82-2-701-8051

FAX: +82-2-701-8058

Email: sample@korea.com

www.CADCAM.co.kr